# Lacal_position_estimator

- 运行过程是什么？

看程序之前先要大概理解卡尔曼算法的原理和实现过程

int local_position_estimator_thread_main(int argc, char *argv[])

{

warnx("starting");

using namespace control;

BlockLocalPositionEstimator est;//构造函数

thread_running = true;

while (!thread_should_exit) {//不断循环

est.update();//数据计算更新

}

warnx("exiting.");

thread_running = false;

return 0;

}

进入BlockLocalPositionEstimator est;构造函数完成矩阵初始化(构造函数是赋初始值的函数)

BlockLocalPositionEstimator::BlockLocalPositionEstimator() :

......

_x(), _u(), _P()

{

......

// initialize P, x, u

initP();

_x.setZero();

_u.setZero();

_flowX = 0;

_flowY = 0;

......

updateParams();

}

进入initP();

- Matrix<float, n_x, n_x>  _P; // state covariance matrix

- enum {X_x = 0, X_y, X_z, X_vx, X_vy, X_vz, X_bx, X_by, X_bz, X_tz, n_x};

- enum {U_ax = 0, U_ay, U_az, n_u};

- enum {Y_baro_z = 0, n_y_baro};

- enum {Y_lidar_z = 0, n_y_lidar};

- enum {Y_flow_x = 0, Y_flow_y, n_y_flow};

- enum {Y_sonar_z = 0, n_y_sonar};

- enum {Y_gps_x = 0, Y_gps_y, Y_gps_z, Y_gps_vx, Y_gps_vy, Y_gps_vz, n_y_gps};

- enum {Y_vision_x = 0, Y_vision_y, Y_vision_z, n_y_vision};

- enum {Y_mocap_x = 0, Y_mocap_y, Y_mocap_z, n_y_mocap};

- enum {POLL_FLOW, POLL_SENSORS, POLL_PARAM, n_poll};

- void BlockLocalPositionEstimator::initP()

- {

- _P.setZero();

- _P(X_x, X_x) = 1;

- _P(X_y, X_y) = 1;

- _P(X_z, X_z) = 1;

- _P(X_vx, X_vx) = 1;

- _P(X_vy, X_vy) = 1;

- _P(X_vz, X_vz) = 1;

- _P(X_bx, X_bx) = 1e-6;

- _P(X_by, X_by) = 1e-6;

- _P(X_bz, X_bz) = 1e-6;

- _P(X_tz, X_tz) = 1;

- }

进入循环est.update()

void BlockLocalPositionEstimator::update()

{

......

// get new data

updateSubscriptions();//获取传感器数据

......

predict();//一步预测，也就是模型预测

if (gpsUpdated) {

if (!_gpsInitialized) {

gpsInit();

} else {

gpsCorrect();//矫正函数

}

}

if (baroUpdated) {

if (!_baroInitialized) {

baroInit();

} else {

baroCorrect();//矫正函数

}

}

if (lidarUpdated) {

if (!_lidarInitialized) {

lidarInit();

} else {

lidarCorrect();//矫正函数

}

}

if (sonarUpdated) {

if (!_sonarInitialized) {

sonarInit();

} else {

sonarCorrect();//矫正函数

}

}

if (flowUpdated) {

if (!_flowInitialized) {

flowInit();

} else {

perf_begin(_loop_perf);// TODO

flowCorrect();//矫正函数

//perf_count(_interval_perf);

perf_end(_loop_perf);

}

}

if (visionUpdated) {

if (!_visionInitialized) {

visionInit();

} else {

visionCorrect();//矫正函数

}

}

if (mocapUpdated) {

if (!_mocapInitialized) {

mocapInit();

} else {

mocapCorrect();//矫正函数

}

}

if (_altHomeInitialized) {

// update all publications if possible

publishLocalPos();//发布主题

publishEstimatorStatus();

if (_canEstimateXY) {

publishGlobalPos();

}

}

......

}

进入updateSubscriptions();

- virtual void updateSubscriptions()

- {

- Block::updateSubscriptions();

- if (getChildren().getHead() != NULL) { updateChildSubscriptions(); }

- }

进入Block::updateSubscriptions();

- void Block::updateSubscriptions()

- {

- uORB::SubscriptionNode *sub = getSubscriptions().getHead();

- int count = 0;

- while (sub != NULL) {

- if (count++ > maxSubscriptionsPerBlock) {

- char name[blockNameLengthMax];

- getName(name, blockNameLengthMax);

- printf("exceeded max subscriptions for block: %s\n", name);

- break;

- }

- sub->update();

- sub = sub->getSibling();

- }

- }

进入sub->update();

这里只会跟到virtual void update() = 0;然后在相对应的.cpp里面找

- void SubscriptionBase::update(void *data)

- {

- if (updated()) {

- int ret = orb_copy(_meta, _handle, data);

- if (ret != PX4_OK) { warnx("orb copy failed"); }

- }

- }

于是通过while (sub != NULL) {}循环，将订阅的主题都copy下来了

接下来要回到est.update()大循环中查看predict()

void BlockLocalPositionEstimator::predict()

{

// if can't update anything, don't propagate

// state or covariance

if (!_canEstimateXY && !_canEstimateZ) { return; }

//输入向量_u

if (_integrate.get() && _sub_att.get().R_valid) {

Matrix3f R_att(_sub_att.get().R);

Vector3f a(_sub_sensor.get().accelerometer_m_s2);

_u = R_att * a;

_u(U_az) += 9.81f; // add g

} else {

_u = Vector3f(0, 0, 0);

}

//动态矩阵A

// dynamics matrix

Matrix<float, n_x, n_x>  A; // state dynamics matrix

A.setZero();

// derivative of position is velocity

A(X_x, X_vx) = 1;

A(X_y, X_vy) = 1;

A(X_z, X_vz) = 1;

// derivative of velocity is accelerometer acceleration

// (in input matrix) - bias (in body frame)

Matrix3f R_att(_sub_att.get().R);

A(X_vx, X_bx) = -R_att(0, 0);

A(X_vx, X_by) = -R_att(0, 1);

A(X_vx, X_bz) = -R_att(0, 2);

A(X_vy, X_bx) = -R_att(1, 0);

A(X_vy, X_by) = -R_att(1, 1);

A(X_vy, X_bz) = -R_att(1, 2);

A(X_vz, X_bx) = -R_att(2, 0);

A(X_vz, X_by) = -R_att(2, 1);

A(X_vz, X_bz) = -R_att(2, 2);

//输入矩阵B

// input matrix

Matrix<float, n_x, n_u>  B; // input matrix

B.setZero();

B(X_vx, U_ax) = 1;

B(X_vy, U_ay) = 1;

B(X_vz, U_az) = 1;

//输入噪声协方差矩阵R

// input noise covariance matrix

Matrix<float, n_u, n_u> R;

R.setZero();

R(U_ax, U_ax) = _accel_xy_stddev.get() * _accel_xy_stddev.get();

R(U_ay, U_ay) = _accel_xy_stddev.get() * _accel_xy_stddev.get();

R(U_az, U_az) = _accel_z_stddev.get() * _accel_z_stddev.get();

//系统过程噪声矩阵Q

// process noise power matrix

Matrix<float, n_x, n_x>  Q;

Q.setZero();

float pn_p_sq = _pn_p_noise_density.get() * _pn_p_noise_density.get();

float pn_v_sq = _pn_v_noise_density.get() * _pn_v_noise_density.get();

Q(X_x, X_x) = pn_p_sq;

Q(X_y, X_y) = pn_p_sq;

Q(X_z, X_z) = pn_p_sq;

Q(X_vx, X_vx) = pn_v_sq;

Q(X_vy, X_vy) = pn_v_sq;

Q(X_vz, X_vz) = pn_v_sq

// technically, the noise is in the body frame,

// but the components are all the same, so

// ignoring for now

float pn_b_sq = _pn_b_noise_density.get() * _pn_b_noise_density.get();

Q(X_bx, X_bx) = pn_b_sq;

Q(X_by, X_by) = pn_b_sq;

Q(X_bz, X_bz) = pn_b_sq;

// terrain random walk noise

float pn_t_sq = _pn_t_noise_density.get() * _pn_t_noise_density.get();

Q(X_tz, X_tz) = pn_t_sq;

//连续时间的卡尔曼滤波器预测值

// continuous time kalman filter prediction

Vector<float, n_x> dx = (A * _x + B * _u) * getDt();

//上一时刻状态推导这一时刻状态线性系统的状态

……

//更新-x状态矩阵，-P协方差矩阵

// propagate

_x += dx;// 这个dx就是通过模型推导

_P += (A * _P + _P * A.transpose() +

B * R * B.transpose() + Q) * getDt();

}

再回到est.update()大循环中查看Correct()，比如flowCorrect();

void BlockLocalPositionEstimator::flowCorrect()

{

// measure flow

Vector<float, n_y_flow> y;

if (flowMeasure(y) != OK) { return; }//获取光流数据

//光流测量矩阵C

// flow measurement matrix and noise matrix

Matrix<float, n_y_flow, n_x> C;

C.setZero();

C(Y_flow_x, X_x) = 1;

C(Y_flow_y, X_y) = 1;

//噪声矩阵R

Matrix<float, n_y_flow, n_y_flow> R;

R.setZero();

R(Y_flow_x, Y_flow_x) =

_flow_xy_stddev.get() * _flow_xy_stddev.get();

R(Y_flow_y, Y_flow_y) =

_flow_xy_stddev.get() * _flow_xy_stddev.get();

//剩余向量r

// residual

Vector<float, 2> r = y - C * _x;//_x来自上面的sonarCorrect()等

//剩余协方差(逆)

// residual covariance, (inverse)

Matrix<float, n_y_flow, n_y_flow> S_I =

inv<float, n_y_flow>(C * _P * C.transpose() + R);//_P来自上面的sonarCorrect()等

//故障检测

// fault detection

float beta = (r.transpose() * (S_I * r))(0, 0);

if (beta > BETA_TABLE[n_y_flow]) {

if (_flowFault < FAULT_MINOR) {

//mavlink_and_console_log_info(&mavlink_log_pub, "[lpe] flow fault,  beta %5.2f", double(beta));

_flowFault = FAULT_MINOR;

}

} else if (_flowFault) {

_flowFault = FAULT_NONE;

//mavlink_and_console_log_info(&mavlink_log_pub, "[lpe] flow OK");

}

//光流矫正

if (_flowFault < fault_lvl_disable) {

Matrix<float, n_x, n_y_flow> K =

_P * C.transpose() * S_I;//卡尔曼增益

_x += K * r; //更新_x, 状态向量

//_x = _x(predict求得)+K(卡尔曼增益) * r(测量与上一状态最优值的误差)

//K =_P * C.transpose() * S_I;

// =_P * C.transpose() * (C * _P * C.transpose() + R)

// =协方差矩阵*光流测量矩阵的转置*(光流测量矩阵*协方差矩阵*光流测量矩阵的转置+噪声矩阵)

// r= y - C * _x

// =光流测量矩阵-测量矩阵*上一状态最优值

_P -= K * C * _P; //更新_P, 状态协方差矩阵

//_P = _P -K * C * _P

//   = (I-K*C)*_P

} else {

// reset flow integral to current estimate of position

// if a fault occurred

_flowX = _x(X_x);

_flowY = _x(X_y);

}

}

到此卡尔曼算法完成！！其实卡尔曼算法就是以下5个公式不断迭代

相同的，高度/GPS，求法是一样的，只是测量矩阵不一样了，噪声等“个性化”东西不一样，处理过程是一样的。

再回到est.update()大循环中发布状态

- if (_altHomeInitialized) {

- // update all publications if possible

- publishLocalPos();

- publishEstimatorStatus();

- if (_canEstimateXY) {

- publishGlobalPos();

- }

- }

publishLocalPos()发布这么多

- _pub_lpos.get().timestamp = _timeStamp;

- _pub_lpos.get().xy_valid = _canEstimateXY;

- _pub_lpos.get().z_valid = _canEstimateZ;

- _pub_lpos.get().v_xy_valid = _canEstimateXY;

- _pub_lpos.get().v_z_valid = _canEstimateZ;

- _pub_lpos.get().x = _x(X_x);    // north

- _pub_lpos.get().y = _x(X_y);    // east

- _pub_lpos.get().z = _x(X_z);    // down

- _pub_lpos.get().vx = _x(X_vx);  // north

- _pub_lpos.get().vy = _x(X_vy);  // east

- _pub_lpos.get().vz = _x(X_vz);  // down

- _pub_lpos.get().yaw = _sub_att.get().yaw;

- _pub_lpos.get().xy_global = _sub_home.get().timestamp != 0; // need home for reference

- _pub_lpos.get().z_global = _baroInitialized;

- _pub_lpos.get().ref_timestamp = _sub_home.get().timestamp;

- _pub_lpos.get().ref_lat = _map_ref.lat_rad * 180 / M_PI;

- _pub_lpos.get().ref_lon = _map_ref.lon_rad * 180 / M_PI;

- _pub_lpos.get().ref_alt = _sub_home.get().alt;

- _pub_lpos.get().dist_bottom = agl();

- _pub_lpos.get().dist_bottom_rate = -_x(X_vz);

- _pub_lpos.get().surface_bottom_timestamp = _timeStamp;

- _pub_lpos.get().dist_bottom_valid = _canEstimateZ;

- _pub_lpos.get().eph = sqrtf(_P(X_x, X_x) + _P(X_y, X_y));

- _pub_lpos.get().epv = sqrtf(_P(X_z, X_z));

publishEstimatorStatus()发布这么多

- _pub_est_status.get().timestamp = _timeStamp;

- for (int i = 0; i < n_x; i++) {

- _pub_est_status.get().states[i] = _x(i);

- _pub_est_status.get().covariances[i] = _P(i, i);

- }   _pub_est_status.get().n_states = n_x;

- _pub_est_status.get().nan_flags = 0;

- _pub_est_status.get().health_flags =

- ((_baroFault > fault_lvl_disable) << SENSOR_BARO)

- + ((_gpsFault > fault_lvl_disable) << SENSOR_GPS)

- + ((_lidarFault > fault_lvl_disable) << SENSOR_LIDAR)

- + ((_flowFault > fault_lvl_disable) << SENSOR_FLOW)

- + ((_sonarFault > fault_lvl_disable) << SENSOR_SONAR)

- + ((_visionFault > fault_lvl_disable) << SENSOR_VISION)

- + ((_mocapFault > fault_lvl_disable) << SENSOR_MOCAP);

- _pub_est_status.get().timeout_flags =

- (_baroInitialized << SENSOR_BARO)

- + (_gpsInitialized << SENSOR_GPS)

- + (_flowInitialized << SENSOR_FLOW)

- + (_lidarInitialized << SENSOR_LIDAR)

- + (_sonarInitialized << SENSOR_SONAR)

- + (_visionInitialized << SENSOR_VISION)

- + (_mocapInitialized << SENSOR_MOCAP);

publishGlobalPos()发布这么多

- _pub_gpos.get().timestamp = _timeStamp;

- _pub_gpos.get().time_utc_usec = _sub_gps.get().time_utc_usec;

- _pub_gpos.get().lat = lat;

- _pub_gpos.get().lon = lon;

- _pub_gpos.get().alt = alt;

- _pub_gpos.get().vel_n = _x(X_vx);

- _pub_gpos.get().vel_e = _x(X_vy);

- _pub_gpos.get().vel_d = _x(X_vz);

- _pub_gpos.get().yaw = _sub_att.get().yaw;

- _pub_gpos.get().eph = sqrtf(_P(X_x, X_x) + _P(X_y, X_y));

- _pub_gpos.get().epv = sqrtf(_P(X_z, X_z));

- _pub_gpos.get().terrain_alt = _altHome - _x(X_tz);

- _pub_gpos.get().terrain_alt_valid = _canEstimateT;

- _pub_gpos.get().dead_reckoning = !_canEstimateXY && !_xyTimeout;

- _pub_gpos.get().pressure_alt = _sub_sensor.get().baro_alt_meter[0];

- 对比inav和lpe算法